E-HAZID

# Assessment Working Draft

ILS857 / ILS-10315 High Throughput Fixed-Bed Reactor Unit

Prepared for multidisciplinary hazard-assessment discussion
17 September 2026

CONTROLLED WORKING DRAFT: This report organizes available evidence and preliminary hazard scenarios. It does not approve operation, declare the assessment complete, or replace accountable engineering judgement. Safeguards are recorded only as context; adequacy, independence, reliability, SIL and residual risk are outside this E-HAZID scope.

| Document control | Value |
| --- | --- |
| Unit / tag | ILS857 / ILS-10315 |
| Draft status | Preliminary, incomplete basis |
| Prepared for | Human multidisciplinary E-HAZID workshop |
| Technical owner / requester | Not identified in the retrieved evidence |
| Primary change considered | Combination: new chemistry, operating window and operating mode |
| Confidentiality | Internal working draft |

# 1. Scope and Change

Assessment boundary. The working scope is the ILS857 / ILS-10315 high-throughput fixed-bed reactor system, including gas and liquid feeds, compression, two four-reactor blocks, downstream separation and collection, gas analysis, venting, sampling, startup, shutdown, regeneration and operator intervention.

Evidence: ILS_BFD.pdf; ILS857 Hazop V2.0.pdf; ILS857-Shell India HT FBR rapid process screening_V4.1_HP_H2.pdf.

## Change classification

New chemistry: the available HAZID register lists naphtha, diesel, heavy ends, oxygenates, hydrogen, oxidic catalysts, steam and nitrogen, with possible CO, aromatics and olefins as side products.

Operating window / mode: the HEMP file contains more than one proposed campaign, including SMPO heavy-ends treatment, naphtha cracking, cracked-gas hydrogenation, acetylene hydroxy-carbonylation and cyclotrimerization. These are not one coherent experiment basis.

Hardware / procedure change: the September 2026 SOP records a new high-pressure hydrogen line under MoC 119975.

INSUFFICIENT BASIS: A single selected experiment, named technical owner, definitive reactant/catalyst/product list, approved operating window, duration, scale, and explicit differences from the established baseline were not identified. Hazard findings below are therefore preliminary and deliberately framed as workshop hypotheses.

# 2. Tier 1 Document Check

| Tier 1 item | Status | Assessment note |
| --- | --- | --- |
| Current P&IDs | Found, revision conflict to resolve | As-built Rev 4.1, issue 05 Aug 2026 was retrieved. Earlier HAZOP used PID V2.1. Confirm Rev 4.1 is workshop master and field-verified. |
| PFD / process description / mass-energy basis | Partly found | BFD and HAZOP process description found. Flow calculations exist, but a campaign-specific mass and energy balance was not identified. |
| Latest SOPs | Found, status unclear | ILS10315 SOP issue Sep 2026 includes HP-H2 line and MoC 119975. Search also found May and July 2026 drafts. Confirm approved controlled copy and coverage of sampling/regeneration. |
| Design limits, MOC, MAWP/MAOP, instrument ranges | Partly found / conflicting | Rev 4.1 drawing states SS316 1/4 inch unless noted and lists P.max 351 bar / T.max 537 C, while the 2020 HAZOP lists section-specific limits up to 50 barg and 700 C. Reconcile component-by-component. |
| SDS for every chemical and catalyst | Missing / unverified | No complete, experiment-specific approved SDS set was established. Generic enterprise SDS search results are not accepted as site-approved copies. |

Evidence: Rev 4.1 as-built P&ID, 05 Aug 2026; ILS857 Hazop V2.0, 09 Mar 2020; ILS10315SOP_Rev3; ILS_BFD; HAZID ILS HTU_NewChemistry; HEMP ID 10315.

## Supporting evidence register

| Document | Revision / date visible | Use in this draft | Status |
| --- | --- | --- | --- |
| ILS857-Shell India HT FBR rapid process screening_V4.1_HP_H2.pdf | Rev 4.1, 05 Aug 2026 | Current as-built process and instrumentation basis | Found |
| ILS10315SOP_Rev3.docx | Issue Sep 2026; version table shows v2 | Operating basis and HP-H2 change | Found, approval state to confirm |
| ILS_BFD.pdf | Date not visible in extracted content | System boundary and major flow path | Found |
| ILS857 Hazop V2.0.pdf | PID V2.1; 09 Mar 2020 | Historical hazards and legacy design basis | Stale for current drawing |
| ILS_ISBL_DSRcloseout.pdf | Review 2020; closeout comments Jan 2022 | Prior scenarios, interfaces and open items | Supporting, conditionally inherited |
| ILS857 IL diagrams V2.7 (1).pdf | Generated 20 Feb 2023 | Documented alarms, ESD and detector actions | Found, revision alignment required |
| HEMP _ID 10315 (ILS).xlsx | Revision/date fields not completed in extract | Candidate chemistry and operating windows | Found, incomplete governance fields |
| HAZID ILS HTU_NewChemistry.xlsx | Date/revision not visible | Prior hazard prompts and actions | Found, incomplete |
| 33.24 ILS857 Flow Calculation.xlsx | Date not visible | Campaign/flow assumptions | Found, exact conditions not frozen |

# 3. Operating Basis

| Parameter | Current / legacy basis | Candidate or proposed basis | E-HAZID disposition |
| --- | --- | --- | --- |
| Reactor arrangement | Eight reactors in two blocks; block-level pressure and temperature control | Same equipment boundary assumed | Confirm field lineup and campaign allocation |
| Reactor temperature | Legacy HAZOP describes heating blocks up to 700 C | HEMP campaigns include <450 C and 300-700 C ranges | Freeze campaign-specific maximum and heat-release basis |
| Pressure | Legacy HAZOP shows many sections up to 50 barg | SMPO 1-35 barg; cracked-gas 9 barg; other campaigns 1-6 barg or about 1 barg | Confirm normal, upset and test pressures by section |
| Gas feeds | H2, C2H2, C2H4, CH4, C2H6, N2, CO, CO2, Ar and O2/N2 are shown | Campaign-dependent mixtures | Approve composition envelope and cylinder identity |
| Liquid feeds | Organic and aqueous feed systems | Naphtha/heavy ends/oxygenates or acetylene solutions depending on campaign | Confirm incompatibility and material compatibility |
| Duration / scale | Not fixed in governing extract | Flow calculation mentions two roughly one-week tests and later agreement of exact conditions | Treat duration and cumulative inventory as open |

# 4. Preliminary Hazard Scenarios

| ID / scenario | Initiating causes | Hazardous event | Consequences / escalation | Exposed | Evidence basis | Confidence | Workshop question |
| --- | --- | --- | --- | --- | --- | --- | --- |
| H-01 Reactive gas composition outside approved envelope | Wrong cylinder, incorrect blend, MFC error, backflow or lineup error | A reactive acetylene/hydrogen/hydrocarbon mixture enters compression or hot equipment | Rapid reaction or decomposition, overtemperature and pressure, loss of containment, fire/explosion | Operators near skid; compressor, KOD, tubing, reactors | Historical DSR/HAZOP; current gas-feed P&ID; prior HAZID notes backflow to MFC | Yellow, inferred for selected campaign | Which exact composition envelope is approved, and how is cylinder identity and lineup verified? |
| H-02 Runaway or severe exotherm in catalyst bed | Higher reactant concentration, excessive catalyst activity, inadequate dilution, loss of temperature control, unexpected side reaction | Heat generation exceeds removal in one or more reactors | Hot spot, accelerated reaction/coking, tube damage, overpressure and downstream release | Operators; reactor block; downstream separator and vent | Prior HAZID explicitly flags exotherm/runaway; HEMP lists candidate chemistries | Red, experiment data gap | Provide reaction calorimetry or justified thermal basis, catalyst identity and worst credible feed composition. |
| H-03 Blocked outlet / coking / liquid accumulation | Heavy feed, partial vaporisation, polymerisation, catalyst fines or coke | Restriction causes rising reactor or upstream pressure and maldistribution | Leak or rupture; hot material release; escalating temperature; contamination across channels | Operators; reactor, capillaries, separators and tubing | Prior HAZID records coking, catalyst deactivation and blocked outlet | Yellow, inferred | How will plugging be detected across individual reactors and during startup/shutdown? |
| H-04 Loss of containment of flammable or toxic gas | Tubing/connection failure, seal incompatibility, maintenance error or vibration | Gas release within enclosure or nearby work area | Fire/explosion, CO exposure, oxygen displacement and domino impact | Operator, maintainer, nearby personnel and adjacent units | Current P&ID lists CO/flammable detectors; IL diagrams record related actions | Green for hazard presence; safeguards not assessed | Confirm detector coverage, ventilation status, simultaneous operations and occupied-area assumptions. |
| H-05 Liquid feed spill or exposure during charging/sampling | Manual transfer, overfill, wrong connection, residual pressure or hot sample | Release of naphtha, heavy ends, oxygenates, solvents or products | Dermal/inhalation exposure, flammable pool/vapour, contamination and waste-generation escalation | Operators, samplers, analytical-lab handlers | Prior HAZID lists 4 L inventory, spillage and sampling; procedures/TRA actions remain | Yellow, inferred | What is the exact maximum connected inventory and approved closed-transfer/sample method? |
| H-06 Utility or ventilation failure | Loss of power, instrument air, N2, cooling, extraction or vent path | Feeds/heat/pressure control or dilution is lost; hazardous material may remain trapped | Release, oxygen-deficient atmosphere, uncontrolled warm-up/cool-down, unsafe recovery | Operators; skid; vent/room systems | IL diagrams list vent monitor and ESD inputs; DSR closeout recorded ventilation partially open in 2022 | Red, current status gap | Has the permanent ventilation action been closed and field-tested for current SIMOPS? |
| H-07 High-pressure hydrogen line interaction | Incorrect isolation, regulator/control failure, backflow, leak or connection to lower-rated segment | High-pressure hydrogen reaches incompatible or lower-rated equipment | Jet release, fire/explosion or mechanical overpressure | Operators; HP-H2 line; compressor and shared headers | Sep 2026 SOP version table identifies new HP-H2 line under MoC 119975 | Yellow, change confirmed; design basis incomplete | Provide MoC closeout, pressure hierarchy, tie-in drawing, test records and operating restrictions. |
| H-08 Regeneration with O2/N2 | Residual fuel/reactants, excessive oxygen, wrong sequencing or insufficient purge | Oxidation begins with a flammable inventory or excessive coke loading | Rapid temperature rise, gas generation, damage and fire | Operators; catalyst beds; heaters; downstream vent | Current P&ID states O2/N2 is only for regeneration | Yellow, inferred | What regeneration recipe, coke loading limit, purge acceptance criterion and abort condition apply? |
| H-09 Incorrect sampling or analytical-lab handoff | Uncertain product profile, poor labeling, residual pressure, incompatible container | Unexpected hazardous sample is opened, transported or stored improperly | Exposure, fire, pressure release or incompatible waste mixing | Skid operator, courier and analytical-lab personnel | HEMP notes uncertain first-experiment product profile and analytical handoff | Yellow, inferred | Who accepts the sample, and what provisional hazard classification applies before analysis? |
| H-10 As-built / document mismatch | Use of V2.1 HAZOP assumptions with Rev 4.1 hardware, incomplete SOP or outdated IL diagrams | Assessment and operating steps omit current line, valve, trip or design limit | Wrong lineup, missed hazard, ineffective emergency response or damage | All operators and equipment | Multiple document revisions and dates retrieved | Red, evidence conflict | Create a signed master-document list and mark up all changes since the 2020 HAZOP. |

# 5. Cross-Discipline Challenges

Document perspective: reconcile Rev 4.1 P&ID, Sep 2026 SOP, 2023 IL diagrams, 2020 HAZOP and 2022 DSR closeout before inheriting assumptions.

Deviation perspective: challenge wrong cylinders, reverse flow to MFCs, cross-contamination, simultaneous campaigns, blocked capillaries, loss of purge, loss of vent and recovery after trips.

Operator perspective: walk through cylinder connection, liquid charging, catalyst loading, leak testing, startup, sampling, draining, shutdown, regeneration, cleaning and maintenance isolation.

Cross-discipline perspective: confirm reaction chemistry, metallurgy and seals, pressure hierarchy, instrument ranges, electrical classification, ventilation interfaces, waste route and human factors.

Safeguard boundary: documented detectors, interlocks, ESD, purges and relief devices are context only. Their adequacy or credited risk reduction must be addressed in authorized specialist reviews.

# 6. Evidence Gaps Register

| Gap | Safety relevance | Suggested owner | Information needed |
| --- | --- | --- | --- |
| Selected experiment and accountable technical owner absent | Prevents a bounded chemistry and operating assessment | Requester / project lead | Experiment ID, owner, purpose, reactants, catalyst, products, phases and schedule |
| Approved SDS set not established | Chemical identity, incompatibility and exposure basis remain incomplete | Technical owner / HSSE | Approved site SDS for every feed, catalyst, solvent, intermediate and expected product |
| P&ID / design-limit reconciliation | Could hide lower-rated components or changed lineups | Process / mechanical | Signed Rev 4.1 master drawing, line list, equipment datasheets, MAWP/MAOP and instrument ranges |
| Thermal reaction basis missing | Runaway and hot-spot scenarios cannot be characterized | Chemistry / process | Calorimetry, heat of reaction, gas-generation potential and sensitivity to feed/catalyst deviations |
| Ventilation closeout status unclear | Release consequences and exposure pathway remain uncertain | Facilities / HSSE | Current test results, flow basis, alarm/trip interfaces and SIMOPS basis |
| SOP approval and task coverage unclear | Manual steps and recovery may not match as-built unit | Operations | Approved controlled SOP covering startup, operation, shutdown, sampling and regeneration |
| MoC 119975 closeout not retrieved | HP-H2 line change may not be fully reflected | MoC owner | Approved MoC, test certificates, marked-up drawings, training and pre-startup checks |
| Campaign duration and cumulative inventory not frozen | Affects cylinder changes, waste, exposure opportunities and thermal history | Requester | Maximum duration, connected inventory, operating cycles and staffing |

# 7. Human Assessment Agenda

1. Confirm scope, experiment ID, technical owner and assessment boundary.

2. Approve the master document register and resolve revision conflicts.

3. Walk down the Rev 4.1 as-built line-up, HP-H2 tie-in, vent and ventilation interfaces.

4. Review chemistry, incompatibilities, exotherm/runaway evidence and possible side products.

5. Challenge each operating phase: charging, startup, steady operation, upset recovery, sampling, shutdown, regeneration, cleaning and maintenance.

6. Review the ten preliminary scenarios and add site-specific causes, escalation paths and exposed groups.

7. Assign owners and due dates for evidence gaps; identify which gaps block further assessment.

8. Route detailed safeguard questions to HAZOP/LOPA/SIS/relief/MoC or other specialist review.

# 8. Specialist Follow-Up

| Review | Trigger from this draft | Expected boundary |
| --- | --- | --- |
| HAZOP revalidation | Rev 4.1 as-built changes, HP-H2 line, new chemistry and operating modes | Detailed deviations, operability and safeguard review |
| Reaction-safety specialist review | Runaway, polymerisation, coke oxidation and gas generation uncertainties | Thermal characterization and safe operating envelope |
| LOPA / SIS | Only if HAZOP identifies scenarios requiring formal risk-reduction verification | Independence, reliability, SIL and risk reduction |
| Relief / depressuring design | Where blocked outlet, runaway, gas generation or pressure-source interaction requires verification | Separate specialist relief basis; PSV sizing is not a Tier 1 E-HAZID requirement |
| Mechanical / materials review | New liquids, solvents, high-pressure H2, seals and coated components | Compatibility, pressure/temperature rating and inspection basis |
| MoC / PSSR | MoC 119975 and any further chemistry, hardware or procedure change | Documented change control and readiness verification |
| Industrial hygiene / ventilation | CO, solvents, oxygen deficiency and enclosure release scenarios | Exposure and ventilation assessment |
| Human factors / TRA | Charging, sampling, recovery, maintenance and analytical handoff | Task-level error traps, ergonomics and work controls |

# Source Notes

Primary internal evidence used: Rev 4.1 as-built P&ID; September 2026 SOP; BFD; 2020 HAZOP; DSR closeout; 2023 interlock diagrams; HEMP ID 10315; new-chemistry HAZID register; ILS857 flow calculation. Citations and source identifiers are listed in the delivery message. This Word file is intended as a workshop-ready working draft, not an approval record.