E-HAZID ASSESSMENT

Heavy Naphtha Reforming | Run18

ILS-10315 | All eight reactors | Human workshop draft

| DOCUMENT CONTROL STATUS Preliminary, evidence-traceable hazard identification draft. The reactor-tube upgrade to a stated 680 degC maximum is recorded as USER-SUPPLIED INFORMATION. The modification has not been verified against an approved certificate, appraisal, MOC close-out, revised PEFS/P&ID, SOP or HEMP. Current controlled documents remain inconsistent. This report does not approve operation, accept risk, assess safeguard adequacy, or replace accountable engineering review. |  |
| --- | --- |
| Field | Value |  |
| Project / experiment | Heavy Naphtha reforming |  |
| Experiment ID | Run18 |  |
| Requester / owner | Harish Balam |  |
| Unit / scope | ILS-10315; R1-R8 |  |
| Proposed envelope | Room temperature / 630 / 650 degC; 1 / 15 / 20 barg |  |
| Stated upgraded tube limit | 680 degC maximum, unverified user statement |  |
| Assessment date | 18 September 2026 |  |
| Planned run date | 01/10/2026 as entered; date convention unresolved |  |
| Output status | Controlled working draft for multidisciplinary review |  |

Prepared by Microsoft 365 Copilot as an evidence-management and hazard-identification assistant. Accountable human review is required.

# Executive decision summary

OVERALL STATUS: INSUFFICIENT BASIS / HOLD AT HUMAN DECISION GATE A full preliminary E-HAZID has been generated, but the evidence set does not support treating 680 degC as an approved unit limit. Run18 proposes 650 degC, which is below the user-stated upgraded tube maximum, but above the 550 degC boundary in the September 2026 SOP and above the 540 degC naphtha-reforming basis in the pre-MOC material. The historical HAZOP says reactor heating blocks can reach 700 degC; that is not the same as a verified pressure-boundary or complete-system operating limit. Resolve document control and component-by-component temperature applicability before authorization.

# Priority findings

F-01 Document conflict: SOP boundary 550 degC versus Run18 maximum 650 degC versus user-stated tube capability 680 degC.

F-02 Upgrade evidence absent: no verified material certificate, wall-thickness/design calculation, pressure-temperature rating, inspection/appraisal, MOC approval or as-built revision for the upgraded tubes.

F-03 System boundary gap: a tube rating does not establish the limits of seals, O-rings, sleeves, reactor heads, valves, thermocouples, fittings, downstream oven, GC path, product collection or vent routing.

F-04 Feed and catalyst identity gaps: the actual heavy-naphtha batch specification/SDS and exact catalyst code/SDS are missing.

F-05 Run plan gap: staging, ramp rates, reduction recipe, H2-to-hydrocarbon ratio, per-reactor allocation, sampling frequency, shutdown/recovery and passivation are not defined.

F-06 Phase-management concern: prior MOC material identifies possible naphtha carryover to vents in fully gas-phase operation and a need for catch-pot/quench arrangements.

F-07 Toxicity concern: BTX, especially benzene, may be present in feed/product; exposure potential concentrates at sampling, collection, disconnection, weighing and waste handling.

F-08 Pressure/flammability concern: continuous pure H2 plus generated H2/light hydrocarbons can create a pressurized flammable inventory; credible leaks include hot reactor connections and downstream sampling/vent paths.

F-09 Catalyst-state concern: fresh oxidic catalyst is reduced in H2, and spent/reduced catalyst may self-heat or react on air ingress; exact catalyst behavior is unknown.

F-10 Long-duration accumulation: 720 h increases opportunity for coke, pressure-drop growth, condensate/dead-leg accumulation, seal degradation, drift and unnoticed vent/catch-pot loading.

# Required accountable decisions before run

| ID | Decision / evidence required | Discipline |
| --- | --- | --- |
| D-01 | Confirm whether 680 degC is an approved tube MAWT/design limit at 20 barg and identify material, dimensions, heat/lot traceability and design code basis. | Mechanical / materials |
| D-02 | Approve or reject 650 degC as the experiment maximum after checking every pressure-boundary and temperature-exposed component. | Technical authority / mechanical / process |
| D-03 | Update or formally redline SOP, PEFS/P&ID, HEMP, equipment appraisal, critical-equipment list and operating envelope. | Document owner / MOC |
| D-04 | Confirm actual feed batch, catalyst code, impurity limits and approved SDS set. | Experiment owner / IH / chemistry |
| D-05 | Confirm downstream phase-control and capacity basis for GC, catch pot, quench/collection and process vent. | Process / operations |
| D-06 | Approve the detailed Run18 run plan, including reduction, startup, sampling, abnormal recovery, shutdown and passivation. | Experiment owner / operations |

# 1. Scope, boundaries and basis

This assessment covers hazard identification for Run18 on ILS-10315, including gas and liquid feeds, all eight fixed-bed reactors, catalyst reduction, startup, heat-up, pressurization, normal operation, sampling, depressurization, shutdown, product collection, venting, catalyst unloading/passivation and maintenance opening. Safeguards are mentioned only as documented context. Safeguard adequacy, HAZOP worksheets, LOPA, alarm/interlock/ESD assessment, SIL, relief sizing and residual-risk judgement are excluded.

# 1.1 Experiment basis

| Parameter | Basis used | Evidence class |
| --- | --- | --- |
| Objective | Convert heavy naphtha to high-octane reformate, aromatics and branched hydrocarbons. | Intake, evidenced |
| Chemistry | Dehydrogenation, dehydrocyclization, isomerization and controlled hydrocracking. | Intake, evidenced |
| Gas feed | 100% H2; maximum 2 NL/min. | Intake, evidenced |
| Liquid feed | Heavy naphtha; broad PIONA ranges; maximum 2 g/h. | Intake, approximate |
| Catalyst | Pt/alumina, possible Re/Ir/Sn and chloride/fluoride; 1 g/reactor; fresh oxidic; reduction planned. | Intake, identity unresolved |
| Reactor scope | R1-R8. | Intake, evidenced |
| Temperature | RT / 630 / 650 degC. | Intake, evidenced |
| Pressure | 1 / 15 / 20 barg. | Intake, evidenced |
| Duration | 720 h maximum. | Intake, evidenced |
| Stated tube upgrade | Maximum 680 degC. Documents not updated. | User statement, unverified |

# 1.2 Screening calculations

These are inventory-throughput screens, not release consequences or relief calculations:

| Screen | Equation | Result | Limitation |
| --- | --- | --- | --- |
| Maximum H2 throughput | 2 NL/min x 60 min/h | 120 NL/h | Does not include H2 produced by reforming. |
| Maximum H2 passed over 720 h | 120 NL/h x 720 h | 86,400 NL | Not simultaneous inventory; actual utilization and interruptions unknown. |
| Maximum liquid feed over 720 h | 2 g/h x 720 h | 1,440 g | Does not include vessel/line hold-up or product inventory. |
| Total catalyst charge | 1 g/reactor x 8 reactors | 8 g | Exact formulation and bulk volume unknown. |
| Nominal gas allocation if equal | 2 NL/min / 8 reactors | 0.25 NL/min/reactor | Actual MFC allocation and operating strategy missing. |
| Nominal liquid allocation if equal | 2 g/h / 8 reactors | 0.25 g/h/reactor | Actual pump split and capillary distribution missing. |

Sanity check: the calculations distinguish cumulative material passed from instantaneous inventory. No room-volume, ventilation-rate, leak-area, effluent-flow or thermochemical values were available, so no dispersion, indoor accumulation, adiabatic temperature rise or pressure-generation result is claimed.

# 2. Evidence register and Tier 1 check

| ID | Document / source | Status | Applicability / conflict |
| --- | --- | --- | --- |
| INT-1 | E-HAZID_Minimal_User_Input_Filled.xlsx | Found | Primary Run18 basis; missing run plan, actual feed spec and catalyst document. |
| INT-2 | SOP_PD-STCB-D-ILS857-01_Sep26.pdf, Rev 3 / Sep 2026 | Conflicting | Covers operations but retains 550 degC unit boundary; does not capture stated 680 degC tube upgrade. |
| INT-3 | NaphthaReforming_ILS_PreMOC Discussion.pptx | Stale / partial | Reforming basis 540 degC, 8-25 barg; SS321H recommended; phase/vent concerns noted. |
| INT-4 | ILS857 Hazop V2.0.pdf, PID V2.1 / 9 Mar 2020 | Stale | Historical design says reactor blocks heated to 700 degC and downstream oven to 200 degC; chemistry and drawings predate current change. |
| INT-5 | ILS857-Shell India HT FBR rapid process screening_V4.0_HP_H2.pdf | Found, applicability unclear | As-built package dated 11 Aug 2022; current upgrade not verified in it. |
| INT-6 | HEMP _ID 10315 (ILS).pdf | Stale / conflicting | Legacy chemistry and <450 degC entries; not Run18-specific. |
| INT-7 | BN Reforming_RG-H2_Slidepack_2025.pptx | Stale / supporting | Gives a prior heavy-naphtha specification and 520/540 degC operating/design context; not the current batch. |
| INT-8 | Heavy Straight Run Naphtha SDS, Version 3.0, 26 Apr 2024 | External/internal search copy; verify applicability | Useful generic hazard reference; not verified as Run18 batch SDS. |
| USR-1 | User statement: reactor tube upgraded to maximum 680 degC; documents not updated | Conflicting / unverified | Treated as a change input and document-control gap, not proof of approved operation. |
| WEB-1 | CDC/NIOSH Pocket Guide: Benzene | External authoritative reference | Supports benzene toxicity/flammability challenge; site occupational limits govern. |

# 2.1 Tier 1 status

| Tier 1 item | Status | Hazard-identification limitation |  |
| --- | --- | --- | --- |
| Current P&ID / PEFS | Conflicting / not demonstrated current | Cannot verify modified tube material, tags, boundaries, vent/catch-pot routing or all exposed components. |  |
| Current process and experiment basis | Partial | Cannot fix reactor-by-reactor conditions, effluent generation, phase split, inventories or abnormal cases. |  |
| Applicable SOPs | Found but stale/conflicting | Sequence context exists, but 650 degC operation is outside the stated 550 degC boundary. |  |
| Hazard-relevant design limits | Missing for modified system | 680 degC tube claim does not establish pressure-temperature rating or adjacent-component limits. |  |
| SDS set | Missing / generic substitutes only | Actual feed/catalyst identity, impurities, exposure and waste properties remain uncertain. |  |
| DOCUMENT UPDATE FLAG Open a controlled document update/MOC action for the reactor-tube upgrade. Until evidence is reconciled, label the system basis as: “Reactor tube reportedly upgraded for maximum 680 degC service. Approved design substantiation and controlled-document incorporation not verified. Do not use this statement as authorization.” |  |  |

# 3. Change classification and assumptions

# 3.1 Change classification

New/changed feed and impurity profile: heavy naphtha batch not yet identified.

Catalyst and catalyst-state change: Pt-based oxidic catalyst with reduction; exact code unknown.

Temperature-window change: 630 degC normal and 650 degC maximum versus 550 degC SOP and 540 degC pre-MOC basis.

Equipment/material configuration change: upgraded reactor tubes reportedly rated to 680 degC.

Operating-mode change: all eight reactors, long-duration operation, sampling and depressurization.

Downstream phase/collection change: high-temperature gas-phase operation with possible naphtha carryover.

# 3.2 Controlled assumptions

| A# | Assumption used for screening | Effect if wrong |
| --- | --- | --- |
| A-01 | 2 NL/min and 2 g/h are total unit maxima. | Per-reactor or total throughput scenarios change. |
| A-02 | 650 degC is reactor set point/wall-related value; catalyst-bed temperature measurement basis is unknown. | Local bed temperature may differ from recorded value. |
| A-03 | No oxidative regeneration is planned in Run18. | Air/oxygen introduction creates additional combustion and thermal scenarios. |
| A-04 | No CO-containing feed is planned. | Metal-carbonyl scenario changes if CO is introduced or generated materially. |
| A-05 | All eight reactors operate in parallel within two four-reactor blocks. | Common-mode and cross-flow scenarios change for another configuration. |
| A-06 | No temporary bypasses or overrides are intended. | Bypasses may invalidate documented context. |

# 4. Four-pass adversarial synthesis

# A. Method and common evidence set

All four passes used the same controlled evidence register in Section 2. Internal search was attempted across files, email, meetings and chats. File results were available; email, meeting and chat retrieval returned service errors, so undocumented discussions could not be confirmed. External literature was used only as generic hazard context and not to replace missing internal design data or SDSs.

# B. Pass 1: Documentation-trusting

| Finding | Initiating cause / event | Consequence / exposure | Confidence / confirm |
| --- | --- | --- | --- |
| P1-01 | Loss of H2 containment from pressurized feed, reactor connection or downstream line. | Flammable cloud, flash fire or jet fire; operators and adjacent equipment exposed. | High; confirm current leak-test boundary and ventilation path. |
| P1-02 | Loss of heavy naphtha/reformate containment during feed, sampling or collection. | Vapour exposure, fire, skin contact, spill and waste impact. | High; confirm actual SDS and collection inventory. |
| P1-03 | High reactor temperature with hydrocarbon/H2 service. | Hot-surface exposure, accelerated degradation, tube failure or ignition after release. | High; confirm complete 650 degC component list. |
| P1-04 | Hydrocracking and dehydrogenation generate/consume gases differently than expected. | Effluent-flow increase, pressure disturbance, light-end loading and vent release. | Medium; mass balance missing. |
| P1-05 | Cooling/condensation differs from design. | Liquid carryover, trap/dead-leg accumulation, sample-bottle overfill or naphtha to vent. | High; prior MOC explicitly flags carryover. |
| P1-06 | Catalyst reduction and later air exposure. | Exotherm, self-heating or pyrophoric response depending on catalyst state. | Medium; exact catalyst datasheet absent. |
| P1-07 | Benzene/BTX in feed or product. | Acute vapour exposure and chronic health concern during open handling. | High generic hazard, unknown concentration. |
| P1-08 | Pressure drop increases from coke, fines or condensation. | Flow maldistribution, hot spots, upstream pressure rise and unstable operation. | Medium; no DP trend basis supplied. |

# C. Pass 2: Skeptic and deviation

| Challenge | Hypothesized deviation | Changed scenario / required check |
| --- | --- | --- |
| P2-01 | 680 degC rating applies only to tube material, not assembled reactor. | Verify heads, seals, sleeves, frits, thermocouple attachment and cold-end temperature. |
| P2-02 | Tube material/heat/geometry differs among R1-R8 or lacks traceability. | Wrong tube installed; inspect markings, certificates and reactor map. |
| P2-03 | SOP/PEFS tags or lineups do not match the post-upgrade field condition. | Incorrect isolation or vent routing; field walkdown and redline required. |
| P2-04 | Temperature indication is wall temperature, not catalyst-bed temperature. | Undetected local hot spot or axial gradient; define measurement basis and uncertainty. |
| P2-05 | MFC conversion factor or gas allocation is wrong for pure H2. | Excess H2 flow, altered residence time and vent load; verify calibration and basis. |
| P2-06 | Liquid capillary maldistribution or partial blockage. | One reactor starved, another overfed; coking, dry-out or carryover. |
| P2-07 | Air ingress after depressurization or during sampling/opening. | Flammable mixture or reduced-catalyst oxidation; verify purge endpoint and opening criteria. |
| P2-08 | Catch pot/chiller/GC arrangement differs from pre-MOC sketch. | Uncontrolled condensation or naphtha discharge to process/local vent; field trace route. |
| P2-09 | Feed contains sulfur, water, chloride, nitrogen or high benzene outside assumed range. | Catalyst poisoning, corrosion, altered product toxicity and unexpected chemistry. |
| P2-10 | Long run causes seal aging, coke growth or instrument drift. | Late-run leak, pressure drop or temperature-control error; define trending and hold points. |

# D. Pass 3: Operational and lifecycle

| Phase | Credible hazard focus | Workshop checks |
| --- | --- | --- |
| Receipt / charging | Wrong feed, static discharge, spill, vapour, incompatible container. | Batch ID, SDS, earthing/bonding need, maximum vessel fill. |
| Catalyst loading | Dust, noble-metal/halide exposure, wrong catalyst, poor packing, frit damage. | Code verification, mass/bed height, PPE/IH basis, reactor map. |
| Reduction | H2 introduction with residual air, exothermic reduction, water generation, temperature overshoot. | Purge criterion, ramp/hold, water handling, endpoint, abort conditions. |
| Heat-up / pressurization | Thermal expansion, seal temperature, wrong sequencing, no/low flow. | Ramp limits, positive flow confirmation, pressure test validity at temperature. |
| Normal operation | Hot H2/hydrocarbon leak, coking, flow maldistribution, gas generation, drift. | Trend variables, material limits, common-mode failure across block. |
| Sampling / collection | Operator proximity, hot/pressurized liquid or gas, benzene exposure, bottle overfill. | Sample conditions, cooling, isolation, ventilation, labeling, waste path. |
| Depressurization | Rapid cooling/JT effect, condensation, aerosol, static, backflow or vent overload. | Ramp, destination, trapped sections, low-temperature compatibility. |
| Shutdown / purge | Residual hydrocarbons, incomplete inerting, reverse flow, blocked vent. | Purge volume basis, endpoint measurement, valve lineup. |
| Opening / unloading | Residual pressure/heat, reduced catalyst oxidation, dust and benzene-bearing residue. | Zero-energy verification, passivation, cool-down and containment. |
| Maintenance / cleaning | Solvent incompatibility, trapped liquid, wrong reassembly, damaged seals. | Cleaning agent SDS, inspection/replacement criteria, post-maintenance leak test. |

# E. Pass 4: Cross-discipline and hidden interactions

| Interface | Interaction / concern | Specialist handoff |
| --- | --- | --- |
| Process - mechanical | 650 degC process target versus unverified assembled pressure-boundary rating. | Mechanical integrity / materials / design authority |
| Process - IH | BTX production and open sampling/collection over a 720 h campaign. | Industrial hygiene / occupational health |
| Chemistry - operations | Reduction/passivation recipe absent for exact catalyst. | Catalyst SME / reaction hazards / operations |
| Process - ventilation | H2 plus hydrocarbon release and possible naphtha carryover to vents. | Ventilation / process safety / facilities |
| Instrumentation - experiment | Wall versus bed temperature; calibration/range and MFC gas correction. | Instrumentation / controls |
| Mechanical - document control | Installed upgrade not reflected in SOP/PEFS/HEMP/appraisal. | MOC / document custodian / inspection |
| Analytical - process | GC and catch-pot path may condense product after analysis. | Analytical / process / operations |
| Environment - operations | Aromatic liquid waste, contaminated catalyst and spills. | Waste / environmental / IH |
| Human factors - operations | Eight parallel reactors and long duration increase lineup and monitoring complexity. | Human factors / operations |

# 5. Reconciled hazard scenarios

| ID | Hazardous event | Cause | Consequence | Phase | Evidence gaps | Priority |
| --- | --- | --- | --- | --- | --- | --- |
| S-01 | Pure H2 leak under pressure | Connection/seal/tube/valve failure; wrong assembly; degradation | H2 release and ignition or indoor accumulation | Feed, reduction, operation, depressurization | Field leak boundary; ventilation rate; release cases | High |
| S-02 | Hot hydrocarbon/H2 release at reactor | Tube rupture, fitting failure, overtemperature, wrong tube | Pressurized hot flammable release inside oven/enclosure | Heat-up, operation | Certificate, wall calculation, component limits, field inspection | High |
| S-03 | Tube service beyond approved assembled limit | 680 degC statement used without complete design substantiation | Creep/strength loss, HTHA concern or seal/head failure | Operation | Mechanical/materials assessment at 650 degC/20 barg | High |
| S-04 | Air-H2 mixture during reduction/startup | Insufficient purge, wrong lineup, air ingress | Flammable mixture heated or pressurized | Reduction/startup | Reduction SOP, purge endpoint, sequence verification | High |
| S-05 | Catalyst-reduction exotherm | Unexpected catalyst composition, fast reduction, water formation | Temperature excursion/local hot spot | Reduction | Catalyst vendor recipe and thermal data | Medium |
| S-06 | Coke formation and reactor plugging | Low H2 ratio, heavy ends, feed impurity, temperature maldistribution | Rising DP and flow redistribution | Operation | DP measurement basis, stop criteria, feed spec | High |
| S-07 | Excess hydrocracking/light-end production | Temperature/high acidity/low residence control | Higher gas generation and flammable effluent | Operation | Mass balance and maximum gas-generation case | Medium |
| S-08 | Naphtha/reformate condensation and carryover | Downstream temperature/cooling/capacity mismatch | Liquid accumulates in GC line, pot, trap or vent | Operation/sampling/shutdown | As-built route and capacity/phase basis | High |
| S-09 | Sample collection exposure | Residual pressure/temperature or incorrect isolation | Release of BTX-containing liquid/vapour | Sampling/collection | Sample inventory, cooling, IH review, exact product SDS | High |
| S-10 | Benzene chronic exposure | Repeated sampling, leaks, waste handling | Cumulative inhalation/dermal exposure | All open handling | Exposure assessment, approved limits, monitoring strategy | High |
| S-11 | Static ignition during liquid transfer | Ungrounded transfer, insulating hose/container, splash filling | Spark ignites naphtha vapour | Charging/waste | Transfer method, conductivity and bonding review | Medium |
| S-12 | Rapid depressurization cooling | Fast blowdown of H2/hydrocarbon mixture | JT cooling, condensation, icing at restrictions | Depressurization | Minimum temperature screen and depressurization profile | Medium |
| S-13 | Reduced/spent catalyst air exposure | Incomplete passivation or premature opening | Self-heating/oxidation, possible ignition | Shutdown/unloading | Passivation procedure and catalyst-specific evidence | High |
| S-14 | Wrong reactor or unequal flow | Valve lineup, MFC mapping, blocked capillary | Some reactors no-flow/overfeed | Startup/operation | Reactor map, calibration, flow confirmation | Medium |
| S-15 | Common-mode block temperature error | Controller/thermocouple placement/calibration error | Four reactors exposed to wrong temperature | Heat-up/operation | Independent verification, sensor placement and test | Medium |
| S-16 | Residual trapped inventory on opening | Dead leg, blocked line, closed valves, incomplete purge | Unexpected hot/pressurized release | Maintenance/opening | Field identification of trapped volumes and verification | High |
| S-17 | Corrosion/halide interaction | Catalyst chloride/fluoride, water, high temperature | Localized attack or stress-related degradation | Reduction/operation | Exact halide/water levels and metallurgy review | Medium |
| S-18 | Volatile metal species | CO contamination/generation with susceptible metals | Potential volatile metal carbonyl/species formation | Operation/shutdown | Confirm CO absence and catalyst/metallurgy susceptibility | Low pending confirmation |
| S-19 | Oxygen deficiency | H2/N2 release or purging displaces air | Reduced oxygen in enclosure/local area | Purge/leak/shutdown | Room/enclosure volume and ventilation verification | Medium |
| S-20 | Long-run drift and unnoticed degradation | 720 h campaign, sensor drift, seal aging, accumulation | Slow approach to unsafe condition | Operation | Monitoring frequency, alarm response outside E-HAZID, inspection hold points | High |

# 6. Topic-by-topic hazard challenge

# 6.1 Chemical reactivity and incompatibility

Actual feed impurity profile is not known. Water, sulfur, nitrogen, olefins and halides may alter catalyst activity, corrosion and coke formation.

Strong oxidizers and air are incompatible with flammable naphtha/H2 service; air ingress is most relevant during startup, shutdown and opening.

Cleaning chemicals are not identified; residue interactions cannot be assessed.

# 6.2 Thermal instability and runaway

Primary reforming reactions are described as heat-consuming, but catalyst reduction, oxidation of reduced/spent catalyst and coke burn-off can be exothermic.

Local catalyst-bed temperature may differ from tube-wall measurement. No thermal-response data, hot-spot margin or adiabatic-rise basis was provided.

No oxidative regeneration is included in the intake; if later added, reassessment is required.

# 6.3 Pressure and gas generation

Dehydrogenation/dehydrocyclization produce H2; hydrocracking produces light hydrocarbons. Maximum effluent generation is not quantified.

Plugging, condensation or closed/incorrect vent routing can increase pressure or redistribute flow.

No relief sizing is performed in this E-HAZID. Confirm existing relief basis separately if the chemistry/gas-generation case changes.

# 6.4 Flammability and explosion

The process intentionally combines H2 and flammable hydrocarbons at elevated pressure and temperature.

Credible ignition sources include hot surfaces, static discharge and electrical/mechanical sources after loss of containment.

No dispersion or explosion calculation is possible without leak sizes, ventilation and enclosure geometry.

# 6.5 Toxicity and exposure

Heavy naphtha and reformate may contain benzene and other aromatics. Actual concentrations are missing.

Priority exposure tasks are feed charging, sampling, bottle changes, analytical transfer, waste handling, opening and catalyst unloading.

The generic heavy-naphtha SDS found by search is not a substitute for the batch-specific approved site SDS.

# 6.6 Phase change and downstream accumulation

Prior MOC evidence explicitly warns that fully gas-phase operation can send naphtha beyond condenser capacity and into process vents.

Condensation after the GC can load a catch pot; low points, dead legs and sampling bottles need field verification.

Dew point and condenser/catch-pot capacity require a traceable mass/phase basis.

# 6.7 Materials and degradation

User states the tube is upgraded to 680 degC, but controlled evidence is absent.

High-temperature assessment must include pressure-temperature strength, H2 service, halides/water, thermal cycling, creep/fatigue and component interfaces.

Historical references to SS316, coated steel, quartz or SS321H are conflicting and cannot establish the installed material.

# 6.8 Catalyst-state hazards

Fresh oxidic catalyst is to be reduced. Exact metal/promoter/halide composition and reduction recipe are unknown.

Reduced or coked catalyst may self-heat on air exposure and may require passivation before unloading.

Carbonyl screening remains conditional because CO feed is not planned but CO absence is not analytically demonstrated.

# 6.9 JT cooling, icing, hydrates and CO2 solids

Rapid H2 depressurization creates cooling potential; hydrocarbon condensation/aerosol formation may occur.

CO2 solidification and hydrate formation are not currently supported as primary scenarios because CO2 feed and water inventory are not defined for Run18.

Retain as conditional checks if CO2, water or wet gas is introduced.

# 6.10 Human factors and simultaneous operation

Eight reactors across two blocks increase verification burden and common-mode exposure.

Long-duration operation creates shift-handover and late-run degradation concerns.

Reactor-to-catalyst map, operating dashboard, clear labels and hold points should be workshop discussion items, not assumed safeguards.

# 7. Worst-case screens and calculation needs

| NO QUANTIFIED CONSEQUENCE CLAIM Available data are insufficient for indoor H2 concentration, flash-fire distance, pressure rise, total effluent rate, hot-spot magnitude, condenser duty or relief response. These remain calculation requests, not hidden assumptions. |  |  |
| --- | --- | --- |
| Case | Bounding definition for specialist calculation | Missing inputs |  |
| WC-01 H2 leak | Largest credible opening in pure-H2 path at maximum upstream pressure and temperature. | Leak diameter/Cv, upstream volume, pressure, isolation response, enclosure/room volume, ventilation. |  |
| WC-02 Reactor tube failure | Full-bore or crack release from one hot reactor at 650 degC and 20 barg. | Tube ID/wall/length, fluid composition, instantaneous inventory, upstream feeding, enclosure geometry. |  |
| WC-03 Maximum gas generation | Most severe credible reforming/hydrocracking product-gas rate from 2 g/h feed plus 120 NL/h H2. | Batch composition, conversion/selectivity bounds, kinetics or validated simulation. |  |
| WC-04 Downstream carryover | Maximum condensable naphtha/reformate entering GC/catch-pot/vent during steady and shutdown cases. | Effluent composition, temperature profile, pressure, condenser/chiller duty, pot volume and drainage. |  |
| WC-05 Reduction thermal case | Fastest credible reduction/oxidation event for 8 g catalyst total. | Exact catalyst, oxidation state, heat release, ramp, H2 concentration, thermal mass and heat-loss data. |  |
| WC-06 Long-run accumulation | 720 h coke/condensate/dead-leg accumulation. | Coke yield, pressure-drop data, low-point volumes, drain frequency and trend history. |  |

# 7.1 Recommended selective checks

Mechanical design substantiation for upgraded tube and assembled reactor at 650 degC/20 barg, with 680 degC maximum claim explicitly addressed.

Maximum product-gas and H2-generation mass balance, including hydrocracking sensitivity.

Phase/dew-point and downstream capacity check through GC, catch pot, quench/collection and vent.

Indoor H2/hydrocarbon accumulation screen using measured ventilation and credible leak cases.

Catalyst reduction/air-exposure thermal screen using vendor data or reaction-hazard testing if data are inadequate.

Temperature measurement uncertainty and wall-to-bed gradient check at the proposed operating range.

# 8. Required actions and workshop handoff

| Priority | Action | Acceptance evidence for workshop | Owner role |
| --- | --- | --- | --- |
| P0 | Create/complete document-control and MOC record for 680 degC reactor-tube upgrade. | Approved change record; scope; design basis; installation/inspection evidence; as-built references. | MOC owner / document custodian |
| P0 | Verify assembled-system materials/design limit at 650 degC and 20 barg. | Signed mechanical/materials statement covering tube, heads, seals, fittings, valves, sensors and hot/cold interfaces. | Mechanical / materials TA |
| P0 | Issue a controlled temporary redline or revised SOP/PEFS/HEMP before authorization. | Approved revision or controlled redline with operating boundary and affected tags. | Operations / process / document owner |
| P0 | Provide Run18 experimental plan. | Reactor map; flows; temperature/pressure sequence; reduction; sampling; shutdown; passivation; abnormal recovery. | Experiment owner |
| P0 | Provide actual feed and catalyst SDS/specifications. | Batch COA/PIONA/boiling range/impurities; catalyst code/composition/reduction guidance. | Experiment owner / chemistry |
| P1 | Confirm downstream gas-liquid routing and capacity. | Field-verified marked-up drawing and calculation basis for condenser/chiller/catch pot/vent. | Process / operations |
| P1 | Define exposure-control review for BTX tasks. | IH task review and monitoring/PPE/containment requirements, outside safeguard adequacy scope. | Industrial hygiene |
| P1 | Define inspection and acceptance of all eight installed tubes. | Tube map, material traceability, dimensional and integrity records. | Inspection / mechanical |
| P1 | Resolve planned run date format. | Unambiguous date written as DD Month YYYY. | Requester |
| P2 | Capture prior operating history and near misses. | Workshop log of leaks, plugging, condensation, sampling issues, tube changes and deviations. | Operators / maintenance |

# 8.1 Workshop decision forks

Fork 1: If approved design substantiation confirms the complete assembled reactor system is suitable at 650 degC/20 barg, retain 650 degC as proposed maximum and update controlled documents. If only the tube is qualified, establish the lower governing component limit or modify the system.

Fork 2: If the current downstream system is verified to keep condensables within designed collection, proceed to detailed operating review. If not, define a modified collection/quench/vent arrangement under MOC.

Fork 3: If actual feed and catalyst identity match reviewed specifications, close generic identity gaps. If not, refresh chemistry, toxicity, corrosion, catalyst-state and waste scenarios.

Fork 4: If a validated reduction/passivation procedure exists for the exact catalyst, use it as workshop evidence. If absent, obtain vendor guidance or specialist testing before finalizing the run basis.

Fork 5: If 01/10/2026 means 1 October 2026, align scheduling accordingly. If it means 10 January 2026, the date is historical and must be corrected.

# 9. Field verification checklist

☐ FV-01  Installed tube material and unique ID for R1-R8

☐ FV-02  Tube OD, ID/wall thickness, length, frit/end closure and certificate traceability

☐ FV-03  Reactor head, sleeve and seal/O-ring materials and temperature exposure

☐ FV-04  Thermocouple type, location, range, calibration and wall/bed interpretation

☐ FV-05  Heating block maximum, uniformity and high-temperature condition

☐ FV-06  Cold-end/O-ring temperature evidence at the proposed reactor temperature

☐ FV-07  All hot-zone valves/fittings/materials and pressure-temperature ratings

☐ FV-08  Current PEFS/P&ID tag match and post-upgrade redlines

☐ FV-09  Hydrogen route, isolation boundary and leak-test scope

☐ FV-10  Liquid-feed path and equal distribution to eight reactors

☐ FV-11  Downstream oven, GC, cold trap/catch pot, quench and product-collection routing

☐ FV-12  Process/local vent destination and low points/dead legs

☐ FV-13  Ventilation availability and measured performance relevant to release screening

☐ FV-14  Sample bottle maximum fill, cooling, labeling and waste transfer

☐ FV-15  No temporary hoses, bypasses, overrides or undocumented connections

☐ FV-16  Previous leaks, plugging, condensation, seal damage or tube replacement history

# 10. Conclusion

Run18 has a credible preliminary hazard basis for multidisciplinary review, but it is not suitable for approval or operation on the current evidence set. The central issue is not simply whether 650 degC is below a stated 680 degC tube limit. The unresolved issue is whether the installed, assembled reactor and connected system have a controlled, approved pressure-temperature-materials basis for 650 degC operation with pure H2 and heavy naphtha.

The user-reported tube upgrade is explicitly flagged as an undocumented change. Existing records conflict: the September 2026 SOP retains a 550 degC operating boundary; pre-MOC naphtha-reforming material uses 540 degC and recommends SS321H; the 2020 HAZOP describes reactor heating blocks up to 700 degC; and legacy HEMP information is for different chemistry and lower temperatures. These sources cannot be silently reconciled.

The human E-HAZID workshop should focus first on the P0 evidence actions in Section 8, then validate the 20 structured scenarios, preserve unresolved disagreement, assign specialist checks and determine the controlled operating basis. No statement in this report constitutes approval, residual-risk acceptance or confirmation of safeguard adequacy.

# Appendix A. Source notes

| ID | Source | Use / limitation |
| --- | --- | --- |
| INT-1 | E-HAZID_Minimal_User_Input_Filled.xlsx | Run18 identity, chemistry, feeds, catalyst, conditions, duration, modes and missing documents. Internal reference: turn2search252. |
| INT-2 | SOP_PD-STCB-D-ILS857-01_Sep26.pdf | Rev 3 includes HP-H2 line; covers startup, leak test, shutdown, depressurization and collection. SOP text states deviations require line-manager approval and search evidence gives ambient-to-550 degC and <30 barg boundary. Internal reference: turn3search593. |
| INT-3 | NaphthaReforming_ILS_PreMOC Discussion.pptx | 540 degC/8-25 barg reforming basis; SS316 unsuitable and SS321H recommended; downstream carryover/catch-pot concerns. Internal reference: turn3search656. |
| INT-4 | ILS857 Hazop V2.0.pdf | PID V2.1, 9 March 2020; eight reactors, two blocks heated to 700 degC and downstream oven to 200 degC. Internal reference: turn3search597. |
| INT-5 | ILS857-Shell India HT FBR rapid process screening_V4.0_HP_H2.pdf | As-built package dated 11 August 2022; used for equipment/topology context only. Internal reference: turn3search600. |
| INT-6 | HEMP _ID 10315 (ILS).pdf | Legacy feed/hazard basis and <450 degC entries; used to show staleness and generic tasks, not Run18 approval. Internal reference: turn3search603. |
| INT-7 | BN Reforming_RG-H2_Slidepack_2025.pptx | Prior heavy-naphtha quality information and comparison showing 520 degC operating and 540/537 degC design context; upgrade was under evaluation. Internal reference: turn3search611. |
| INT-8 | XNS6TV.pdf - Heavy Straight Run Naphtha SDS, version 3.0, 26 April 2024 | Generic hazard classification located in enterprise search; applicability to Run18 batch not verified. Internal reference: turn3search810. |
| WEB-1 | CDC/NIOSH Pocket Guide to Chemical Hazards - Benzene | CAS 71-43-2, occupational hazard and flammability context. Accessed 18 September 2026. Web reference: turn3search717. |
| WEB-2 | Vitol Naphtha SDS US V3 | Generic naphtha flammability, aspiration, irritation, carcinogenicity and static-control context. Applicability not verified. Web reference: turn3search716. |

# Appendix B. Evidence classification legend

| Class | Meaning |
| --- | --- |
| Evidenced | Directly stated in a retrieved controlled or supplied source. |
| User-supplied, unverified | Provided in conversation but not supported by retrieved controlled documentation. |
| Inferred | Engineering hypothesis used to challenge the basis; requires human confirmation. |
| Gap | Needed information not found or not supplied. |
| Conflicting | Two or more sources or inputs disagree; no silent resolution. |
| Stale | Document predates chemistry, configuration or modification and has limited applicability. |